# SUNPOR — SIGNALS & ML GUIDE

Line E10 Extrusion | How signals work, machine areas, and meanings

# HOW TO READ A SIGNAL NAME

Setpoint (Sollwert)      Target the system wants

Actual (Istwert)         Real measured value now

Control output (Stell)   Heater PID command (%); can be negative — normal

Error code (Fehler)      Feeder fault flags (overload, comm error, etc.)

Operating mode           Feeder running state (GD, VR, VS, LF, EF, …)

Throughput / quantity    Flow rate (kg/h) or accumulated weight (kg)

# MACHINE AREAS ON THE LINE

Physical zones where sensors are mounted. Signals are grouped by area, then

used together to understand what the machine is doing.

AREA                        | SIGNALS | WHAT THIS PART OF THE LINE DOES

## |---------|

Extruder Heating Zones      |   27    | Heats barrel along extruder length

Melt Pressure               |    5    | Measures melt pressure & temperature

Extruder + Melt Pump        |    4    | Screw + pump speed and load (torque)

Screen Changer + Die Plate  |   14    | Filter melt, startup valve, die holes

Water Box / Process Water   |    7    | Cools strands before granulator

Granulator                  |    6    | Cuts foam strands into beads

Feeder (Doser)              |   31    | Doses EPS, graphite, recycled, etc.

Pentane / Nitrogen          |    3    | Foaming gas dosing

Antistatic                  |    2    | Additive for bead surface/cells

Off-spec / Scrap            |    5    | Scrap counters by destination/size

Material Production         |    1    | Flag: line producing (1) or not (0)

TOTAL: 104 WinCC signals — all from SignalExport.xlsx

# ALL SIGNALS BY MACHINE AREA (name — meaning)

## EXTRUDER HEATING ZONES (27)

Heats plastic before it is pushed through the extruder. Zones 1–7 are cylinder

zones; zone 8 is flange; zone 9 is material discharge section.

Cylinder Zone 1–7 Temperature Setpoint     Target temp per zone (°C)

Cylinder Zone 1–7 Temperature Actual       Measured temp per zone (°C)

Cylinder Zone 1–7 Temperature Control Out  Heater power command (%)

Flange Temperature Setpoint / Actual / Control Output

Material Discharge Section Temp Setpoint / Actual / Control Output

Related to: melt temperature, startup (warming up), stable production

Watch: setpoint vs actual gap; control output negative = heater idle (OK)

## MELT PRESSURE (5)

Measures resistance of molten material flowing through the extruder.

Extruder Melt Pressure 1–4                 Pressure at 4 points (bar)

Extruder Melt Temperature 1                Melt temp (°C)

Related to: screen changer, nozzle, blockages, process stability

Watch: sudden rise = blockage risk; drop during empty run is normal

## EXTRUDER + MELT PUMP (4)

Extruder screw melts and pushes material; melt pump controls flow to die.

Extruder Screw Speed                       Screw rotation (rpm)

Extruder Torque                            Screw load (%)

Melt Pump Speed                            Pump rotation (rpm)

Melt Pump Torque                           Pump load (%)

Related to: throughput, material viscosity, low production

Watch: torque up + throughput down = material or blockage problem

## SCREEN CHANGER + STARTUP VALLE + DIE PLATE (14)

Screen filters contaminants; startup valve controls start-up flow; die plate

(Lochplatte) has holes where foam exits.

Screen Changer Inlet Melt Pressure         Pressure before screen (bar)

Screen Changer Outlet Melt Pressure        Pressure after screen (bar)

Startup Valve Melt Pressure                Pressure at startup valve (bar)

Startup Valve Housing Temperature          Valve body temp (°C)

Startup Valve Melt Temperature             Melt temp at valve (°C)

Screen Changer Zone 1 / 2 Temperature      Screen heater zones (°C)

Screen Changer Adapter Temperature         Adapter temp (°C)

Die Plate 1–4 Temperature                  Each die zone temp (°C)

Related to: screen clog, nozzle problems, open holes in die

Watch: inlet minus outlet pressure rising = screen blocked

## WATER BOX / PROCESS WATER (7)

Cools extruded strands in water bath before granulator cuts them.

Water Box Process Water Pressure P1 / P2   Water pressure points (bar)

Process Water Temperature Actual / Setpoint Cooling water temp (°C)

Water Pump Speed                           Pump speed (%)

Process Water Flow Actual / Setpoint       Water flow (m³/h)

Related to: bead shape, twin beads, lump formation, grain size

Watch: flow or temp off setpoint → cutting/cooling problems

## GRANULATOR (6)

Rotating knife cuts cooled strands into EPS beads.

Granulator Speed Actual / Setpoint         Cutter speed (rpm)

Granulator Torque                          Motor load (%)

Knife Position                             Knife position (mm)

Granulator Hydraulic Contact Force Actual  Knife press force (%)

Granulator Hydraulic Contact Force Setpoint Target press force (%)

Related to: grain distribution, sieve analysis, off-spec coarse/fine

Watch: torque rising over time = knife getting dull

## FEEDER / DOSER (31)

Dosers (Dosierer) weigh and feed different materials into the extruder.

Total Throughput Setpoint / Actual         Total line output (kg/h)

Feeder 1 (EPS) Throughput Actual / Setpoint        Main polystyrene

Feeder 2 (Graphite) Throughput Actual / Setpoint   Graphite additive

Feeder 3 (Recycled) Throughput Actual / Setpoint   Recycled material

Feeder 4 (600) Throughput Actual / Setpoint        Material type 600

Feeder 5 (Off-spec) Throughput Actual / Setpoint   Recycled scrap back in

Feeder 6 (Pentane) Throughput Setpoint           Pentane dosing target

Feeder 7 (Nitrogen) Throughput Setpoint            Nitrogen dosing target

Feeder 1–5 Speed Actual                    Feeder screw speed (%)

Feeder 1–7 Error Code                      Fault bits per feeder

Feeder 1–5 Operating Mode                  Feeder state code

Related to: recipe mix, low production, material behavior, pentane balance

Watch: actual far below setpoint = dosing problem; error code ≠ 0 = fault

Note: Feeder 6/7 have setpoint only in export (no actual throughput tag).

## PENTANE / NITROGEN (3)

Pentane = blowing agent for foam. Nitrogen = process gas for cell structure.

Pentane Quantity Actual                    Pentane flow (kg/h)

Nitrogen Quantity Actual                   Nitrogen flow (kg/h)

Pentane Pump Frequency Actual              Pump speed (Hz)

Related to: foaming behavior, too much/little pentane, cell structure

Watch: pentane per kg throughput vs normal for material type

## ANTISTATIC (2)

Additive sprayed for bead surface and processing.

Antistatic Quantity Actual / Setpoint      Flow (l/h)

Related to: foaming, surface quality

## OFF-SPEC / SCRAP (5)

Counters for reject material — NOT the same as Feeder 5 which doses scrap in.

Off-spec Quantity Scrap Line               Scrap to waste line (kg)

Off-spec Quantity Octabin                  Scrap to octabin (kg)

Off-spec Quantity Silo                     Scrap to silo (kg)

Off-spec Quantity Sieve Coarse             Oversize beads rejected (kg)

Off-spec Quantity Sieve Fine               Undersize beads rejected (kg)

Related to: quality problems, granulator, sieve analysis

Watch: counters climbing fast = quality getting worse

## STATUS (1)

Material Production Status (1 = Producing)   1 = line in production, 0 = not

Related to: knowing if line should have throughput or is stopped

HOW SIGNALS WORK TOGETHER (RELATIONSHIPS)

Temperature zones (1–9)  →  Is machine hot enough and stable?

Melt pressure 1–4        →  Is melt flowing freely?

Screen in vs out pressure →  Is screen clogged?

Die plate temps          →  Are holes open and even? (open holes quality)

Total throughput         →  How much product per hour?

Feeder actual vs setpoint →  Is recipe mix correct?

Pentane + nitrogen       →  Is foaming agent correct?

Process water flow/temp  →  Is cooling OK for bead shape?

Granulator torque/force  →  Is knife cutting well?

Off-spec coarse/fine     →  Is bead size distribution bad?

Material production = 1  →  Should we expect full production values?

Example — screen blockage building:

Screen inlet pressure ↑  +  outlet pressure ↓  +  melt pressure ↑

Example — dull knife:

Granulator torque ↑  +  off-spec coarse ↑  +  contact force drifting

Example — pentane problem:

Pentane actual ↓ vs setpoint  +  poor pressure stability  +  weak foaming

# ML MODELS — HOW SIGNALS ARE USED

Signals are grouped (heating_zones, feeders, melt_pressure, etc.) and converted

to features (mean, std, trend over time windows). Each model uses specific groups.

## SIGNAL GROUP → WHICH MODELS USE IT

heating_zones      → Process State, Anomaly

feeders            → Process State, Low Production, Anomaly

extruder_meltpump  → Process State, Low Production, Anomaly

melt_pressure      → Process State, Nozzle/Screen, Predictive Quality

screen_changer     → Nozzle/Screen, Anomaly

process_water      → Anomaly

granulator         → Granulator/Knife, Anomaly

pentane_nitrogen   → Material Behavior Risk, Anomaly

offspec            → Predictive Quality, Anomaly

status             → Process State, Low Production

Build order: Model 1 first — all other models need to know production phase.

## MODEL 1 — PROCESS STATE (build first)

Goal: Recognize which production phase the line is in.

Phases:

Heating up, Startup, Stable production, Low production,

Cleaning run, Empty run, Cooling down, Shutdown, Fault

Main signals used:

Total throughput, extruder speed/torque, melt pressure/temp,

heating zones, melt pump, feeder operating modes, material production status

How it works:

Feature engine computes trends and averages over time window.

Rule engine (rules_config.yaml) matches patterns to a phase.

Stable production: CONFIRMED on real Line E10 data.

Other phases: ESTIMATED — need more historical phase data to calibrate.

Example patterns:

Heating up     → temps rising, throughput zero, extruder slow

Stable run     → throughput ~setpoint, status=1, pressures stable

Empty run      → extruder running, throughput zero, low pressure

Fault          → feeder errors, extreme pressure variance

Why first: A pressure drop in empty run is normal; in stable run it is a problem.

## MODEL 2 — LOW PRODUCTION

Goal: Detect when line runs below target throughput (not startup/shutdown).

Main signals:

Total throughput actual vs setpoint, feeder actual/setpoint,

extruder speed/torque, melt pressure, operating modes

Output:

Low production yes/no, duration, intensity, possible cause

(startup, shutdown, material problem, feeder fault, planned state)

Why: Low production must not be trained as bad stable production.

## MODEL 3 — MATERIAL BEHAVIOR RISK

Goal: Predict developing material problems before they worsen.

Operator labels (training):

Lump formation, twin beads, outside sieve tolerance,

too little/much pentane, poor foaming behavior

Main signals:

Temperature profile, pressure profile, throughput, feeder proportions,

pentane/nitrogen, process water, granulator, material type, production phase

Output:

Risk % per material problem type, affected area

Signal patterns watched:

Twin beads        → water flow/temp off, granulator deviation

Pentane issues    → pentane actual vs expected ratio wrong

Lump formation    → water flow deviation, granulator torque up

## MODEL 4 — PREDICTIVE QUALITY (needs enough operator labels)

Goal: Predict later bad quality before lab results confirm it.

Operator labels (targets):

Open holes %, sieve distribution %, foaming OK/NOK, material blocking,

cell structure, wrong recipe, twin beads

Main signals:

Full stable-phase data: melt pressure, off-spec counters, granulator,

pentane, feeders, water, screen/nozzle temps

Plus: maintenance events, material behavior entries

Output:

Quality risk per run, foaming risk, sieve risk, open holes risk,

material blocking risk

Signal patterns watched:

Open holes risk   → die plate temp unstable, melt pressure rising

Sieve risk        → off-spec coarse/fine rising, granulator torque trend

Foaming risk      → pentane deviation, pressure instability

## MODEL 5 — GRANULATOR / KNIFE

Goal: Predict knife maintenance need and grain distribution problems.

Operator labels:

Knife change, knife sharpening, poor sieve analysis, off-spec coarse/fine

Main signals:

Granulator speed/torque, knife position, contact force actual/setpoint,

process water, off-spec coarse/fine

Output:

Knife condition normal/abnormal, grain distribution risk,

maintenance recommendation

Signal patterns watched:

Knife dull        → torque trend up, contact force drifting

Cutting problem   → off-spec coarse up, speed deviation

## MODEL 6 — NOZZLE / SCREEN / PRESSURE

Goal: Detect nozzle blockage, screen clog, pressure problems.

Operator labels:

Screen change, nozzle change/flushing/grinding, nozzle blockage,

chiseling free, open holes, material block

Main signals:

Melt pressure 1–4, screen changer inlet/outlet pressure,

startup valve pressure, die plate temps, melt temp,

extruder torque, melt pump torque

Output:

Pressure problem yes/no, possible cause (screen / nozzle / blockage)

Signal patterns watched:

Screen blocked    → inlet/outlet pressure difference rising

Nozzle problem    → startup valve pressure up, die plate temp uneven

## HOW THE SYSTEM DETECTS QUALITY PROBLEMS EARLY

Uses signal TRENDS over time windows (not single readings):

1 min, 5 min, 15 min, 30 min, full production run

Before lab results:

Watch pentane ratio, water flow, granulator torque, pressure trends,

off-spec counter rate, feeder deviations

Operator material behavior entries confirm patterns and train models.

Daily quality + material blocking = ground truth for predictive quality.

## HOW THE SYSTEM LINKS PRODUCTION + MATERIAL + MAINTENANCE + QUALITY

Production run (material type, shift)     = context container

Machine signals                           = physical process state

Operator events (knife/nozzle/screen)     = maintenance actions

Material behavior                         = early symptoms on floor

Daily quality                             = measured outcome

Material blocking                         = confirmed bad batch later

Off-spec counters                         = scrap accumulating live

ML connects: "During 753 run, granulator torque rose + twin beads logged

+ sieve coarse increased" → future similar pattern triggers warning.

## WARNINGS BEFORE SCRAP (ML OUTPUT TYPES)

Stored as ml_predictions:

process_state           Current phase + confidence

low_production          Yes/no, duration, cause

quality_risk            Overall quality risk per run

material_behavior_risk  Risk per problem (lump, pentane, twin beads…)

granulator_risk         Knife condition, grain distribution risk

pressure_problem_risk   Screen / nozzle / blockage risk

maintenance_risk        Knife or nozzle maintenance hint

Warnings only apply during STABLE PRODUCTION — not startup/cleaning/empty run.

# GLOSSARY — TERMS YOU WILL HEAR

Doser / Feeder (Dosierer)

Weighing/feeding unit that doses material into extruder (EPS, graphite,

recycled, pentane, etc.). Feeder 1–7 on Line E10.

Dosing behavior

How feeders run compared to target: actual vs setpoint, speeds, mix %,

errors, operating modes. Wrong dosing → pressure and quality problems.

Throughput

Total production rate through the line (kg/h). Main signal for how much

product is being made. Compared to setpoint for low production detection.

Contact force (Anpresskraft)

Hydraulic force pressing granulator knife against cutting surface (%).

Too low or drifting = poor cut or worn knife.

Temperature profile

Pattern of temperatures across all heating zones — not one sensor alone.

Shows if heating is balanced and stable along the extruder.

Pressure profile

Pattern of melt pressures at multiple points. Difference between points

(e.g. screen inlet vs outlet) shows where resistance/blockage is.

Off-spec

Material that fails quality specification (scrap/reject). Signals count

accumulated scrap (kg). Feeder 5 doses recycled off-spec back into mix.

Screen changer (Siebwechsler)

Filter unit in melt line. Screen clogs over time → pressure rises → needs

screen change (operator event).

Die plate / Lochplatte

Plate with holes where foam exits extruder. Temperature affects open holes.

Pentane

Blowing agent that creates foam cells. Too little or too much → bad foaming.

Nitrogen

Process gas affecting cell structure. Works with pentane for foam quality.

Twin beads

Two beads stuck together — often water box / cooling / cutting issue.

Lump formation

Clumps in product — often water flow, granulator, or cutting issue.

Sieve analysis

Lab test: bead size distribution through sieves. Bad result = poor grain

distribution. Linked to granulator and off-spec coarse/fine counters.

Open holes

Percentage of open holes in die plate — daily quality measurement.

Blocked holes → pressure and die temperature patterns change.

Setpoint vs Actual

Setpoint = what system wants. Actual = what is measured. Large sustained

gap = problem (dosing, temperature, flow).

Control output (negative values)

PID heater command in %. Negative (e.g. -100%) means zone is at temperature

and heater is not demanding power. This is NORMAL — not bad data.

Snapshot

One moment in time: all 104 signals read together, same timestamp.

OPERATOR INPUTS (NOT MACHINE SIGNALS — FOR CONTEXT)

Operators type these in forms. They are NOT WinCC tags but explain what signals

mean (e.g. "empty run" explains why throughput is zero).

Production start     Material type (750, 753…), trial, shift

Extruder events      Heating, startup, shutdown, screen/nozzle work

Granulator/knife     Knife change or sharpening

Cleaning             Water basin, centrifuge

Faults               Blockages, power failure

Material behavior    Lump, twin beads, pentane issues, poor foaming (early)

Material blocking    Batch held later for quality (confirmed bad)

Daily quality        Open holes %, sieve distribution %, foaming OK/NOK

# END OF GUIDE

Reference: SignalExport.xlsx (104 tags), Extrusion Line E10